How to Update Your

ZombieVerter Dial

(Over WiFi — No Cables Needed!)

The 10-Minute Guide

# What is this?

Your ZombieVerter Dial is a tiny computer in your car's dash. Every now and then, a new version of its software comes out with bug fixes and new features. This guide shows you how to install the latest version using just your phone — no plugging it into a laptop, no programming, nothing scary.

If you can install an app on your phone, you can do this. The whole thing takes about 10 minutes.

# What you'll need

- Your car (parked somewhere safe)

- Your phone OR a laptop

- Internet on your phone for ONE minute (to download the update file)

- The PIN code for your dial

Heads up Once you start the actual update, the dial needs to stay powered on for about 30 seconds. Don't turn the car off mid-update. If the power dies during an update, the dial can get stuck and would need a USB cable to fix.

# Part 1: Get the update file

First we need to download the new software onto your phone. It's a small file — under 2 MB.

- On your phone, open a web browser (Safari, Chrome, whatever you use).

- Go to this address: https://github.com/robertwa1974/Zombieverter-Dial-Display/releases

- You'll see a list of releases. The newest one is at the top with a green "Latest" tag next to it.

- Tap or click on the latest release to open it.

- Scroll down to the section called Assets. Tap the file named firmware.bin to download it.

Important: get the RIGHT file You want firmware.bin. NOT factory.bin, NOT "Source code (zip)", NOT "Source code (tar.gz)". Only firmware.bin works for the wireless update. If you grab the wrong file, the update will fail.

Your phone will save firmware.bin to its Downloads folder (or ask where you want it). Just remember where it went — you'll need to find it again in a couple of minutes.

# Part 2: Wake up the dial's WiFi

Your dial has a hidden trick: it can act like a little WiFi router. We're going to turn that on so your phone can talk to it directly.

- Sit in your car and turn the ignition on so the dial powers up. (The engine/motor doesn't need to actually be running — just enough power for the dial.)

- If the dial is locked (showing the padlock screen), enter your PIN to unlock it.

- Once you see the normal dashboard, press and hold the dial button for about 2 seconds.

- The screen will change to show WiFi info — a network name, a password, and a web address. That means it worked.

Didn't work? If holding the button doesn't bring up WiFi mode, try a short press first (you might already be in WiFi mode and another press would turn it off). The screen should clearly say something about WiFi when it's on.

# Part 3: Connect your phone to the dial

Now we tell your phone to join the dial's WiFi network. This is just like connecting to coffee shop WiFi.

- On your phone, open Settings → WiFi.

- Look for a network called ZombieVerter-Display and tap it.

- When it asks for a password, type: zombieverter (all lowercase, one word)

- Your phone might warn you that this network has no internet. That's normal — tap "Stay Connected" or "Connect Anyway" or whatever the option is to keep using it.

iPhone users — read this! iPhones are sneaky. If you have other WiFi networks saved (like your home WiFi), your iPhone might secretly jump back to one of those during the update because the dial's network has no internet. Before starting the update, go into Settings → WiFi → tap the little (i) next to your OTHER known networks, and turn OFF "Auto-Join." You can turn them back on after.

# Part 4: Do the update

Almost done! Now we just send the file to the dial.

- Stay connected to the ZombieVerter-Display WiFi network.

- Open your phone's web browser and go to this address: http://192.168.4.1/ota.html

- You'll see an update page with a big box that says you can drop a file there.

- Tap the box (or the button on it) to open your phone's file picker. Find firmware.bin in your Downloads folder and select it.

- A progress bar will appear and start filling up. Don't touch anything for 20-30 seconds.

- When it's done, the dial will reboot automatically. You'll see it go blank for a few seconds, then come back to life on the lock screen.

- Enter your PIN to unlock. You're now running the new version!

DO NOT do these things during the update • Don't turn off the car • Don't unplug anything • Don't switch WiFi networks on your phone • Don't close the browser tab • Don't poke the dial buttons Just sit there for 30 seconds and let it finish. Easy.

# Part 5: Check it worked

After the dial reboots and you unlock it, you can verify the new version is installed.

- Rotate the dial to find the Settings screen — it usually shows the firmware version number.

- Compare it to the version number on the GitHub release page. Match? Done!

If you turned off Auto-Join on your other WiFi networks earlier, you can turn them back on now.

# Help! Something went wrong

## My phone won't connect to ZombieVerter-Display

- Double-check the password — it's all lowercase: zombieverter

- Make sure the dial actually shows the WiFi info screen. If not, hold the button again.

- Turn your phone's WiFi off and back on, then try again.

## The web page won't load

- Make sure you typed the address exactly: http://192.168.4.1/ota.html

- It must start with http:// not https:// (that little 's' will break it)

- Check the top of your phone screen — are you still connected to ZombieVerter-Display? If your phone jumped back to LTE or another WiFi, it can't see the dial.

## The progress bar froze or said "failed"

- Make sure you uploaded firmware.bin, not factory.bin or a source code zip.

- Check the dial — is it still powered on? Did the screen change?

- Try again from the beginning. The dial isn't broken — failed uploads don't damage anything as long as power stayed on.

## The dial rebooted but the screen is weird / blank

- Wait a full minute. Sometimes the first boot after an update takes longer.

- Turn the ignition off, wait 10 seconds, turn it back on.

- If it still won't come back, you'll need to plug it into a laptop with USB. Contact whoever set this up for you.

## I don't see firmware.bin on the release page

If the latest release ONLY has "Source code (zip)" and "Source code (tar.gz)" but no firmware.bin, the person who made the release forgot to attach it. Send them a message and ask them to add it — you can't do the wireless update without that file.

# That's it!

You just updated firmware on an embedded device over a wireless connection. That's a real engineering thing. Pretty cool, right?

Your settings, your PIN, and your saved RFID cards all stay the same after an update — nothing gets erased. The only thing that changes is the software itself.

Drive safe.